# Zero-Copy Vision — Part 3: The Library

pycamtrt as it stands at v0.4.0, and what it measures against DeepStream

Part 3 of the Zero-Copy Vision series. Part 1 describes the silicon-up pipeline: one frame, one stream, decoded and inferred without a byte crossing to host memory. Part 2 measures that pipeline against the world — a DeepStream head-to-head, the operating envelope across four resolutions, tiled inference — and produces the capacity atlas a fleet deployment is sized from. This part describes pycamtrt as it stands at v0.4.0: an installable, declarative Python library, import pycamtrt, with the C++ core underneath it. Every number in it was measured on the machine the README’s versions section names, an RTX 3060 Ti, and every claim has a gate that can be re-run.

## Contents

- Chapter 1 — Preface: From Pipeline to Library

  - 1.1 What the engine is

  - 1.2 What it measures against

  - 1.3 The thesis

  - 1.4 The governing principle, and why it cannot bend

  - 1.5 What this book covers

- Chapter 2 — The Two Planes

  - 2.1 The declarative API

  - 2.2 From object graph to fused conductor

  - 2.3 Honest rejection

  - 2.4 The result path

- Chapter 3 — Design Rulings and the Lessons Behind Them

  - 3.1 The API rulings, and the linked list that was rejected

  - 3.2 The core as a library, and what gates catch that compilers cannot

  - 3.3 Lifecycle guarantees

  - 3.4 The binding

  - 3.5 Born-verified parity

  - 3.6 Five defects the gates caught

- Chapter 4 — The Proof

  - 4.1 Against a naive Python baseline

  - 4.2 Against the rebuilt modern conventional stack

  - 4.3 Against DeepStream 8.0

  - 4.4 The pycamtrt scaling curve

  - 4.5 The soak

  - 4.6 Honest framing

- Chapter 5 — The Endpoint

  - 5.1 The insight

  - 5.2 The PacketRing

  - 5.3 Sinks: tees that cannot backpressure

  - 5.4 The answer table

  - 5.5 Measured: zero cost, at scale and over time

- Chapter 6 — Any Model, Any Tree

  - 6.1 Norm and color: a scalar becomes per-channel

  - 6.2 Four families, one contract

  - 6.3 The recipe

  - 6.4 yolo-e2e: detection without NMS

  - 6.5 The depth-2 tree

  - 6.6 The classifier-parity gate

- Chapter 7 — Making an Atlas

  - 7.1 Capacity as a function, latency as its outcome

  - 7.2 How a campaign is run

  - 7.3 The fitted models

  - 7.4 recommend(): the atlas surfaced in the API

  - 7.5 How a user makes an atlas for their own GPU

- Chapter 8 — The Decoder Path and the Ceiling

  - 8.1 How a frame reaches the batch

  - 8.2 Where the ceiling is

  - 8.3 The dials against DeepStream

  - 8.4 The atlas fits and the current decoder

  - 8.5 What the ceiling teaches

- Chapter 9 — Where It Stands, What’s Next

  - 9.1 What exists, and its gate

  - 9.2 The honest gaps

  - 9.3 The roadmap, in order

  - 9.4 The repository and its versions

  - 9.5 Afterword: one method

# Chapter 1 — Preface: From Pipeline to Library

## 1.1 What the engine is

The engine under pycamtrt is the pipeline of Part 1. A 720p H.264 stream is demuxed on the CPU, decoded on NVDEC straight into VRAM, preprocessed by one fused kernel, run through a TensorRT engine, and postprocessed on the GPU — box decode and NMS both. One frame closes end to end in about 2 ms, verified bit-exact against a CPU reference. Multiplied into a fleet, the same design runs one producer thread per camera feeding one shared GPU thread and one shared CUDA context, a greedy batcher pooling frames across every camera, and a plate-reading cascade cropped straight out of a full-resolution decode ring.

## 1.2 What it measures against

Part 2 measured that engine against DeepStream 8.0 with a custom bounding-box parser and a custom CTC parser on the DeepStream side, so both systems ran the same verified decode math. The engine led on post-decode latency at sixteen cameras and used less VRAM. Part 2 also mapped the operating envelope from 720p to 4K, found the limiting resource moving from decode silicon to memory as pixel counts grow while inference cost stays flat, and spent that resolution-invariance on SAHI, GPU-resident tiled inference verified against a planted-object CPU reference. Chapters 4 and 8 repeat the DeepStream comparison on the library itself, at fleet scale and on one night; where the numbers differ from Part 2’s, the newer ones stand.

## 1.3 The thesis

Proving the engine competitive was the precondition, not the goal. The target is an open-source, modular Python library for cascade inference over many concurrent RTSP streams — a DeepStream alternative. Open where DeepStream is closed. Pythonic where DeepStream is GStreamer-shaped. Aimed at the same surveillance and edge workloads DeepStream serves, but described in a few lines of Python instead of a plugin graph and a config-file dialect, and usable by someone who never reads a line of the CUDA underneath.

## 1.4 The governing principle, and why it cannot bend

One rule sits above every design decision: Python is the control plane, C++ is the data plane, and the two never blur. Python describes a processing graph — which engines, in what order, feeding what crops to what — and the C++ core executes that graph, fused, once, at full frame rate. No Python object, function or callback is ever invoked on the per-frame path. Pixels are born in VRAM and stay there; only the compact survivors of a batch — a few dozen boxes, a handful of strings — reach Python, and only after the GPU has done the reducing.

The reason this cannot bend is not stylistic. It is the entire measured gap between this library and every alternative it has been benchmarked against. A naive Python pipeline — OpenCV capture, torch inference, nothing exotic — measures 12.01 ms/frame against the engine’s 2.28, purely from ordinary Python objects sitting where the data plane sits. That tax is constant at one stream and stops being constant at scale: a modern conventional stack (OpenCV + ultralytics-ONNX + PaddleOCR + PyQt5) measures 13.6 ms at one stream and 87.6 ms at eight — a 210% duty cycle, over real time — while the GPU underneath it sits at 40% the whole time. The wall is Python and CPU orchestration: per-thread model instances, a GIL, context switches, all doing at eight streams what one shared engine call does at sixteen. One Python callback on the per-frame path would not cost a constant few milliseconds; it would reproduce that collapse, because it puts the interpreter back where the conventional stack never left. The principle is the mechanism the speed advantage runs on.

## 1.5 What this book covers

Chapter 2 describes the two planes as pycamtrt exposes them: the declarative object API a user writes, and the compile step that turns it into the fused conductor the GPU thread runs. Chapter 3 records the design rulings behind that API and the lifecycle guarantees the core makes, with the defects that shaped them. Chapter 4 is the measurement: four opponents plus a soak. Chapter 5 is the endpoint — how data gets back out without a copy tax. Chapter 6 is the family system that makes the library model-agnostic. Chapter 7 is the capacity atlas and the planner built from it. Chapter 8 describes the decoder path, where the fleet ceiling is and why, and what the library measures against DeepStream on every dial. Chapter 9 closes with what exists, its gates, and its gaps.

Takeaway. The library is a proof turned into an interface: three object kinds a user writes, never a per-frame callback, because the callback is exactly what the conventional-stack comparison shows costs 6–25x at scale.

Cost so far. The numbers this chapter cites (2.28 ms, 12.01 ms naive, 87.6 ms conventional-stack collapse) are the engine’s; Chapter 4 checks the library against them.

# Chapter 2 — The Two Planes

## 2.1 The declarative API

pycamtrt’s Python surface is deliberately small: three generic step kinds, grouped into named layers, wired together by reference. Process is an explicit preprocess stage. Engine runs a TensorRT engine on its input. Postprocess decodes that engine’s raw tensor into detections (family="yolo" or "yolo-e2e"), text ("ctc"), or a classifier label ("argmax"). None of these is a task class. There is no OCR object anywhere in the API — plate reading is the running example throughout this book series, never an API concept, because the library is model-agnostic by construction: swap the engine path and the family string, and the same three step kinds build a make/model classifier instead.

Every step’s constructor takes its input as a direct object reference — Engine(det, "ocr.engine"), not a name, not a position in a list. That reference is the single source of truth for the whole graph: execution order is derived from it (a topological sort), never maintained by hand alongside it. Layer("read").add(step) returns the step as a handle for later steps to point at; layer.steps is a read-only view in insertion order. When an Engine in one layer takes a Postprocess step from an earlier layer as its input, that cross-layer edge auto-inserts the crop — the Python-level expression of the C++ core’s ring -> CropParams -> child-engine-input hop. The user never calls a crop function; naming the edge is enough.

A minimal cascade, real syntax, unmodified from the package’s own docstring:

import pycamtrt

streams = pycamtrt.Streams(["rtsp://cam1", "rtsp://cam2"])
L1 = pycamtrt.Layer("detect")
eng = L1.add(pycamtrt.Engine(streams, "yolov8n_b1-16_fp32_sm86.engine"))
det = L1.add(pycamtrt.Postprocess(eng, family="yolo", score=0.4, iou=0.45))

L2 = pycamtrt.Layer("read")
eng2 = L2.add(pycamtrt.Engine(det, "ocr.engine"))     # edge => auto-crop
txt = L2.add(pycamtrt.Postprocess(eng2, family="ctc"))

with pycamtrt.Pipeline(streams, layers=[L1, L2]) as pipe:
    for r in pipe:
        print(r.stream_id, r.frame_no, r.outputs["read"])

Six lines describe a two-model cascade across two cameras. None of them decodes a frame, moves a pixel, or crops an image — that is the C++ side’s job, and it starts only when Pipeline compiles this object graph.

## 2.2 From object graph to fused conductor

Python object graph        index graph               ExecPlan            fused conductor
 Streams / Layer / Step  ->  StepDesc / LayerDesc   ->  Validate()      ->   Batcher, Nv12Ring,
 (dataflow references,      (PipelineConfig, plain     (pipeline.cpp:      producers, GPU
  held by identity)          integer indices)           shape check,       loop, SAHI, cascade
                                                         resolves norm/     (moved verbatim
                                                         color defaults)    Chapter 8)

Pipeline.__init__ walks every layer’s steps in insertion order, keyed by Python object identity: the Streams object is seeded into that identity map as index -1 (the C++ convention for “raw stream source”), and every step compiles into a StepDesc whose input field is another step’s already-compiled index. A step referring to an object not yet seen in the graph — added out of dependency order — fails immediately with a ValueError, in Python, before any C++ is touched. That is the index graph: the same dataflow the user wrote, now flattened to integers a C++ struct can hold.

Construction of the extension module’s _c.Pipeline(cfg) is where the two planes meet. Under a released GIL, the C++ side loads and warms up every engine, then calls Validate(cfg) (pipeline.cpp) — a function that both checks and returns an ExecPlan: the one shape, out of everything the generic graph could describe, that the executor actually knows how to run. That shape is a depth-2 tree — one Engine -> Postprocess("yolo"|"yolo-e2e") root, optionally tiled by SAHI, feeding any number of sibling recognition layers, each its own Engine -> Postprocess("ctc"|"argmax") cropping the same root. Validate() resolves per-layer norm/color defaults into that plan, and only then does the fused conductor — the same Batcher, refcounted Nv12Ring, producer threads, and GPU loop that Chapter 8 walks through frame by frame — start running it.

## 2.3 Honest rejection

Every shape the generic API can express but the executor cannot run is rejected by name, not silently reinterpreted. An unrecognized family= string raises ValueError listing every valid one. Feeding one recognition layer’s own Postprocess output into another Engine — chaining a child off a child, rather than two siblings off the same root — is accepted at the Python level (the object graph has no opinion) and rejected by Validate() with a named RuntimeError: “v1 executor supports a depth-2 tree: one detector layer feeding sibling recognition layers; chained cascades (a child of a child) are not yet executable.” Attaching SAHI to a cascade layer instead of the detector layer raises its own named error the same way. The rule is consistent across all three: a shape the API can describe but the executor cannot run fails loudly, at construction, in a sentence naming exactly what was asked for and why it does not fit — never a silent fallback to some other behavior.

## 2.4 The result path

Once started, the GPU thread emits one FrameResult per completed frame into a bounded queue — capacity 256 by default (queue_capacity), backpressure="block" unless the user opts into "drop_oldest" (with dropped_results() reporting evictions). Python’s for r in pipe: calls poll(500) in a loop, timeout in milliseconds, GIL released for the wait so Ctrl-C and other threads stay responsive; a timeout just loops back around rather than raising. Every result carries the same three-way latency split a Sink(kind="events") line mirrors verbatim into its NDJSON: ms_pre (pop-to-ready), ms_queue (ready-to-take), ms_gpu (take-to-done) — attached to every result a consumer ever sees, whether that consumer is a Python for loop or a TCP socket. Chapter 8 says exactly where each clock starts and stops.

Takeaway. Two planes meet at exactly one seam: the ExecPlan Validate() returns at construction. Above the seam is Python objects and identity-keyed dictionaries; below it is the fused conductor, and no per-frame call ever crosses back over.

Cost so far. One compile pass at construction time (engine load + warmup, done once); zero Python calls per frame afterward. The result queue holds 256 FrameResults by default before backpressure engages.

# Chapter 3 — Design Rulings and the Lessons Behind Them

## 3.1 The API rulings, and the linked list that was rejected

The API’s shape was settled by explicit ruling before any binding code existed, and the rulings still describe it. Package name pycamtrt; the core keeps its own identity (pycamtrt_core). Generic stages, not task classes — “OCR” is an example throughout this book, never an API concept. Dataflow references as the single source of truth; Layer.add(step) returns a handle; layer.steps is read-only.

One alternative was rejected and the reason is part of the design: a linked-list API, step[i] with step i’s input implicitly step i-1. It fails on two counts. It creates two sources of truth — a step’s position in the list and its actual data dependency — that must be kept in agreement by hand forever, exactly the duplication the reference-based design exists to avoid. And a linear chain cannot express a tree: the moment two recognition layers must crop the same detector output — a plate reader and a make/model classifier both reading one root, which is what Chapter 6’s sibling children do — a chain has no syntax for two steps sharing a parent. Explicit input=<step> references solve both for free: order is derived by topological sort, and a tree is two steps pointing at the same upstream object.

## 3.2 The core as a library, and what gates catch that compilers cannot

src/core/{graph.h, result.h, pipeline.h, pipeline.cpp} hold the step-graph PipelineConfig, FrameResult and pycamtrt::Pipeline. The conductor — Batcher, the refcounted Nv12Ring, producer threads, the GPU loop, SAHI serial and pooled, the OCR cascade, --verify — is the engine’s, moved into the library unchanged, explanatory comments included. Validate() maps the generic graph onto the executor and rejects anything else by name. The reference CLI, rtsp_infer_multi, is written on top of the same library and the same graph API a Python user writes, with identical output; the atlas of Chapter 7 drives that binary.

The house rule the whole project runs on was learned when that CLI was first rewritten on the library: it compiled cleanly, linked cleanly, and hung, because it built the graph, polled in a loop, and never called Start() — perfectly well-typed, and no compiler could object. Only the gate caught it: a --verify run against 400 reference frames that simply never produced one. The checkpoint gates catch what compilation cannot, and nothing in this library ships without one.

## 3.3 Lifecycle guarantees

Every construct, start, poll and stop ordering a user can hit — including the adversarial ones — has a defined behavior, and each was a real defect before it became a guarantee:

- A constructor that throws leaks nothing: partial allocations are released on the throwing path, where a destructor never runs.

- Concurrent Stop() calls are safe: the losing thread blocks on stop_seq_complete until the winner’s joins have finished, so no destructor can free CUDA state mid-join.

- Start() after Stop() raises instead of spawning threads nobody would join, which crashed via std::terminate.

- The stream-info statistics cache is safe under concurrent readers and writers.

pipeline_api_test is the gate: a construct → start → consume → stop matrix across every teardown variant (double-Stop, destructor-only, poll before Start), a hang watchdog, and a VRAM-growth detector, run for five to twenty cycles with zero growth. The reconnect gate matters most for a network-facing library: kill and restart a live stream mid-run. The producer backs off from 250 ms to a 5 s cap, retries, catches up, and the run closes with reconnects=1, failed=0 on the affected stream and zero non-monotonic timestamps on every stream that was never touched.

## 3.4 The binding

_pycamtrt, a small pybind11 module, is thin on purpose. The GIL is released around every blocking C++ call — constructor, start, poll, stop — so a Python thread never holds the interpreter while a GPU thread warms an engine or a producer blocks on a socket. A log-callback trampoline is the one place that acquires the GIL deliberately, routing any exception raised inside a user’s log= callback to sys.unraisablehook rather than letting it escape into C++. python/pycamtrt/__init__.py is the declarative API plus the object-graph-to-index-graph compiler of Chapter 2.

One rule of the binding exists because of a deadlock. Garbage-collecting an unstopped Pipeline with a Python log= callback attached deadlocked: pybind11 runs the C++ destructor under the GIL, a worker thread could at that moment be blocked acquiring the GIL inside the trampoline, and the destructor’s joins then waited on a worker that could never proceed. The object’s __del__ therefore calls the GIL-releasing stop() before the C++ destructor can run, so every worker is joined — and no callback can be mid-flight — before anything executes under the GIL. The gate: a live SIGINT mid-stream with a real Python callback attached exits with status 0, no hang.

## 3.5 Born-verified parity

examples/read_plates/read_plates.py is the program the library exists for: an fp16 plate detector feeding an LPRNet cascade, live plate prints and a run summary, with a --verify mode. The parity gate runs the C++ CLI and the Python library against identical engines and identical content and compares outputs: the dominant plate string is identical from both, plates per frame 1.00 against 1.00, and the Python side’s own verify=True closes at 600/600. The library agrees with the reference implementation it wraps, and the gate that says so is re-run on every change to the core.

## 3.6 Five defects the gates caught

| # | Where | Defect | Severity | Rule that came out of it |
| --- | --- | --- | --- | --- |
| 1 | Nv12Ring | A ring slot could be overwritten while a queued batch still referenced it, letting a crop come from a frame newer than its own detections | Correctness (silent) | Per-slot in_use refcount; ring slot acquired before the batch slot; Ensure() waits for all-free before reallocating |
| 2 | Reference CLI | Never called Start(); compiled clean, hung forever on poll timeouts | Blocker | Gates, not compilers, decide when something works |
| 3 | Core Stop() | Concurrent stops could let a destructor free CUDA state mid-join | Use-after-free | Losers block until full quiescence |
| 4 | Core Start() | Start() after Stop() spawned threads nobody joined | Crash | stop_requested guard raises instead |
| 5 | Python wrapper | GC of an unstopped pipeline with a log= callback deadlocked destructor and trampoline on the GIL | Deadlock | __del__ stops before the destructor runs |

A sixth belongs with Chapter 8: a map that waited on the inference stream, which no gate here could see because every gate ran below the camera count where it bit.

Takeaway. None of the defects that shaped the library was an algorithm mistake. Every one was ownership, lifecycle, or Python-object-model correctness, surfacing once a verified C++ core had to live under a garbage collector and a GIL — and every one is now a guarantee with a gate.

Cost so far. Zero Python calls per frame; a compile pass at construction; a lifecycle matrix and a reconnect gate that close at zero growth and zero failures.

# Chapter 4 — The Proof

Four comparisons, four different opponents, plus a soak, establish what the library holds against and where it stops.

## 4.1 Against a naive Python baseline

Naive OpenCV-plus-torch, one process per camera, no batching, no shared context: 12.01 ms/frame, next to the engine’s 2.28 ms — a more than 5x gap, and the floor every other comparison in this chapter sits above. Memory shows the sharper version of the same architectural difference: the naive design scales RAM roughly linearly, about 1.3 GB per stream, because every camera is its own process with its own model copy. pycamtrt’s fleet shares one CUDA context and one engine across every camera; the marginal cost of one more stream is a decode surface and a batch slot, not a model — measured at roughly +20 MiB per added camera.

## 4.2 Against the rebuilt modern conventional stack

Python-RTSP/cascade_baseline.py: OpenCV RTSP capture, ultralytics ONNX plate detection (verified GPU via onnxruntime CUDA), PaddleOCR 3.7, optional PyQt5 rendering — modern versions, deliberately untuned but competent, run on the same farm and the same plate content as pycamtrt.

| Streams | Conventional stack | pycamtrt | Factor |
| --- | --- | --- | --- |
| 1 | 13.62 ms (33% duty @ 24 fps) | 2.24 ms | 6.1x |
| 4 | 42.87 ms (103% duty — over real time) | 2.57 ms | 16.7x |
| 8 | 87.63 ms (210% duty) | 3.45 ms | 25x |

The shape, not just the numbers, is the finding: the conventional stack’s aggregate throughput plateaus around 86 fps — going from 4 to 8 streams bought zero additional throughput, only doubled latency — while the GPU underneath it sits flat at roughly 40% utilization the entire time. The wall is Python and CPU orchestration (sixteen per-thread model instances oversubscribing the host), not compute. pycamtrt holds the full offered rate at every stream count measured, with the GPU as the only limit in sight. Memory shows the same shape: 4.1 GB RAM / 2.5 GB VRAM at eight streams for the conventional stack versus roughly 0.5 GB RAM / 1 GB VRAM at sixteen for pycamtrt; a marginal camera cost the conventional stack about +200 MB RAM and +240 MiB VRAM against pycamtrt’s ~+20 MiB.

The conventional stack is built honestly — not a strawman, real production libraries at current versions — and each of the costs of doing so is a finding in its own right:

| # | Finding |
| --- | --- |
| 1 | Paddle’s pip install silently broke torch, twice (dependency clobbering) |
| 2 | Stable paddlepaddle-gpu 3.0 cannot run PaddleOCR 3.7’s models — nightly required |
| 3 | onnxruntime silently falls back to CPU without LD_LIBRARY_PATH surgery |
| 4 | ultralytics serializes all inference threads through one lock |
| 5 | PaddleOCR’s recognizer silently returned another thread’s answer 21% of the time under shared use |
| 6 | PyQt5 + paddle crash on the wrong import order |
| 7 | +5.6 GB of dependencies pulled in |
| 8 | Both stacks read 100% of plates, but PaddleOCR’s string flickers frame-to-frame where LPRNet’s fixed-format decode holds steady |

## 4.3 Against DeepStream 8.0

Measured on one night, same clip, same farm, same hour, every frame of every camera decoded and inferred at 30 fps, the plate cascade on both sides, DeepStream tuned per NVIDIA’s latency guidance (fp16 engines, 5 ms batch timeout, 30 ms jitter buffer):

| held at ≥95% of the offered rate | pycamtrt | DeepStream tuned fp16 | DeepStream default fp32 |
| --- | --- | --- | --- |
| 720p | 48 cameras (end of grid), 9.3 ms | 48, 12.7 ms | 24, 63 ms |
| 1080p | 24, 3.3 ms | 24, 10.2 ms | 24, 82 ms |
| 4K | 6, 1.8 ms | 6, 8.6 ms | 6, 17.9 ms |

Post-decode latency at every count where both hold, pycamtrt against tuned DeepStream: 720p 2.8 / 3.4 / 4.6 / 6.0 / 9.3 ms against 8.1 / 8.1 / 8.6 / 11.5 / 12.7 at 16 through 48 cameras; 1080p 2.5 / 2.7 / 3.0 / 3.3 against 8.3 / 8.3 / 8.3 / 10.2 at 8 through 24; 4K 1.9 / 1.9 / 1.8 against 5.7 / 8.4 / 8.6 at 2 through 6. The two are equal with one camera, where inference alone sets the number; two to four times apart below the wall; and equal again on the wall, where both queue at whatever rate the decoder allows. Chapter 8 explains the mechanism and shows that on every dial setting the two hold the same number of cameras — the ceiling is the silicon, and the difference is how long a frame waits below it. VRAM at fp16 and sixteen cameras: about 10% leaner than DeepStream.

## 4.4 The pycamtrt scaling curve

The full sweep of the Python binding itself, not the underlying C++ CLI:

1 stream  : 2.24 ms
  4 streams : 2.57 ms
  8 streams : 3.45 ms
 16 streams : 4.77 ms   (both sinks attached: events + live RTSP relay)

At sixteen streams the binding held 419.1 results/s — exactly the full offered rate — with the events sink delivering 4,800 of 4,800 expected NDJSON lines and zero drops. The curve is flat and shallow: latency roughly doubles from 1 to 16 streams while a conventional stack’s latency at just 4 to 8 streams already doubles on its own with zero throughput gain (§4.2). The control-plane/data-plane split — Python describing, C++ executing — holds all the way to fleet scale, sinks included.

## 4.5 The soak

Sixteen streams, both sinks attached, skip=2, run for 3.5 hours (12,602 s):

| Gate | Result |
| --- | --- |
| Throughput | 3,023,750 results at 239.9/s — exactly the offered rate, every minute of 3.5 h |
| Memory drift | RSS varied 827->830 MB — a 3 MB band across the entire run; VRAM flat ~1.68 GB after warmup |
| Delivery accounting | events == results exactly (3,023,750 = 3,023,750); sink_dropped=0, dropped_results=0 |
| Stream health | reconnects=0, failed=0 across all 16 streams |
| Relay consumability | 14/14 quarter-hourly ffprobe checks returned valid h264/1280 |
| Clean stop | exit 0, full teardown, GPU idle after |

Three million results, zero dropped events, zero drift worth measuring — the endpoint capability and the library underneath it are soak-proven at fleet scale, not just proven on a five-minute run.

## 4.6 Honest framing

pycamtrt is faster than every alternative it has been measured against at the one job all four comparisons test: RTSP in, cascade inference out, at low and flat latency. It holds as many cameras as DeepStream’s tuned configuration and no more, because both stop at the hardware decoder. It is not feature-complete against the DeepStream suite. There is no tracker — a skipped frame carries no interpolated boxes, only the next inferred frame’s fresh ones. There is no annotated-video endpoint — Sink forwards raw compressed video or NDJSON metadata, never drawn pixels; that is NVENC work already scoped. Tier-2/3 frame access (hold_frames=True) is treated as experimental: an exit-time crash seen in earlier internal builds has not reproduced across the lifecycle gates since, but its write site was never localized. Packaging is pip install . from source inside the documented container, not a wheel. None of these change what this chapter’s numbers say about the core task; they are the honest list of what “not a DeepStream replacement yet, but faster at what it does” means in practice.

Takeaway. Four opponents, four different angles, one shape: pycamtrt holds a flat 2–5 ms band from 1 to 16 streams while naive Python, a modern conventional stack, and tuned DeepStream each cost more, scale worse, or both; at fleet scale it holds the same cameras as tuned DeepStream at a third of the latency. The soak turns that into a claim about hours, not one lucky run.

Cost so far. 2.24->4.77 ms across 1->16 streams (sinks on at 16); 3,023,750 soaked results, 0 drops, a 3 MB RSS band over 3.5 h; 2–4x lower latency and ~10% less VRAM than tuned DeepStream below the wall; 6.1–25x ahead of the conventional stack at 1–8 streams.

# Chapter 5 — The Endpoint

## 5.1 The insight

The pipeline keeps pixels off the CPU: NVDEC decodes into VRAM, a fused kernel writes straight into the inference binding, nothing round-trips through host RAM on the per-frame path. Getting video back out looks at first like it should break that rule — decode, draw, re-encode, copy down. It doesn’t. The compressed frame already crosses host RAM once, for free, before NVDEC ever touches it: FFmpegDemuxer hands packets to the producer thread on the CPU side of the boundary, and Decode() is called on that same memory. Forwarding the original video downstream costs nothing new — no re-encode, no device-to-host copy — because the bytes were already there.

## 5.2 The PacketRing

src/core/packet_ring.h gives each stream a host-side ring of its last ring_seconds of demuxed packets (default 10 s, ≈2.4 MB/stream at 720p). The producer thread memcpy’s each packet into the ring before calling Decode(), so the ring never depends on decode succeeding. Two properties make it useful rather than just cheap: it’s keyframe-anchored (replay from any point in the ring starts at a real IDR, so it’s always decodable) and PTS-indexed (so a ring position lines up with the detection that came from that same frame). ring_seconds=0 disables the ring entirely — no allocation cost paid for a pipeline that never needs it.

## 5.3 Sinks: tees that cannot backpressure

A Sink is a tap on the graph, not a stage in it. Each sink drains through its own bounded, drop-oldest queue on a dedicated thread — a stalled consumer loses its own oldest data (sink_dropped() counts it) and never slows the pipeline that produced it. Three things flow out this way:

producer -> ring (packets) --------> Sink(kind="stream")  -> RTSP relay
       |                                Pipeline.clip()      -> .h264 file
       v
   GPU thread -> FrameResult -------->  Sink(kind="events")  -> NDJSON

kind="events" emits one NDJSON line per FrameResult (stream, frame, pts, boxes, texts, the pre/queue/gpu latency split, a printable frame_addr) to tcp://, file://, or stdout. kind="stream" republishes the original compressed H.264 live, -c copy semantics through mediamtx — a real muxer, no re-encode anywhere. A relay’s first connection replays the ring’s history at real-time pace before going live, so a viewer who joins mid-run gets continuous context instead of a jump to “now.” pipe.clip(stream, seconds_back, path) pulls the same ring into a pre-roll .h264 file, snapshot-and-fwrite, no sink required.

Two restrictions are deliberate, not oversights. One relay sink serves exactly one concrete stream — one RTSP path is one camera; multiplexing several onto one path has no defined behavior, so stream= is required, not optional, for kind="stream". And the events format string is built only when an events sink actually exists — a pipeline that never configures one pays nothing for the formatting code sitting in the binary. Because RTSP-fed packets already arrive Annex-B, the framing NvDecoder expects, clip files are plain writes with no re-framing step; the demuxer handles the AVCC framing of MP4 file inputs itself.

## 5.4 The answer table

The question the endpoint answers is simple: does sending frames back out require compressing them again?

| Output | Re-encode needed? | Mechanism |
| --- | --- | --- |
| Original video | No | PacketRing relay, -c copy |
| Annotated video | Yes | pixels exist only in VRAM — the NVENC sink, not yet built |
| Metadata (events) | No | already compact, formatted once on the GPU thread |

Annotated video is the one case that can’t dodge a re-encode: once boxes are drawn onto the frame, that frame exists only as pixels in VRAM, and getting compressed bytes back out means running an encoder. That sink doesn’t exist yet; the Sink graph node built here is the base it will attach to.

## 5.5 Measured: zero cost, at scale and over time

A 15-cell grid (N ∈ {4, 8, 16} × {off, ring, events, stream, both}) measured end-to-end latency with every sink combination on, gated at ≤5% overhead and zero drops:

off      ring     events   stream   both
  N=4   2.748ms  2.811    2.755    2.706    2.741
  N=8   3.323    3.284    3.254    3.244    3.254
  N=16  4.278    4.338    4.359    4.164    4.040

Every delta sits inside the ±5–10% run-to-run noise band — several sink cells measured below their own off sibling — and sink drops were zero in all 15 cells. All five predictions stated before the grid ran hold. At fleet scale with both sinks attached, 16 streams: 419.1 results/s (the full offered rate) at 4.77 ms end-to-end, 4800/4800 events delivered, 0 dropped. That 4.77 ms sits exactly on the Python binding’s own established scaling curve with no sinks at all — 1 stream: 2.24 ms, 4: 2.57, 8: 3.45, 16: 4.77 — so turning both sinks on did not add a new point to that curve, it landed on the one already there.

The soak of §4.5 pushes past a single grid: 16 streams, both sinks, 12,602 s (3.5 h). 3,023,750 results at exactly the offered rate (239.9/s) every single minute; RSS held in an 827→830 MB band; events delivered exactly matched results delivered (3,023,750 = 3,023,750, sink_dropped=0); all 14 quarter-hourly relay probes came back decodable h264. Three million results, zero dropped events, no drift — the endpoint is soak-proven, not just grid-proven.

Takeaway. The endpoint costs nothing because it is designed around a fact already true of the pipeline — compressed packets already sit in host RAM — rather than adding a new copy to move them. Fifteen grid cells and a 3.5-hour, 3-million-result soak put the sink cost indistinguishable from zero.

# Chapter 6 — Any Model, Any Tree

## 6.1 Norm and color: a scalar becomes per-channel

Every Engine step applies (pixel[c] + offset[c]) * scale[c] before its model sees a frame, inside the fused preprocess kernel. The knob is per-channel, ((o0,o1,o2), (s0,s1,s2)) (a scalar pair broadcasts to all three), with a load-bearing convention: index 0/1/2 mean the first/second/third channel in this engine’s own channel order — whatever color="rgb" or color="bgr" says it is. That convention is what lets a user copy a model’s published mean/std directly: torchvision states ImageNet’s mean as [0.485, 0.456, 0.406], RGB-ordered, which converts to this knob as offset[c] = -mean[c]*255, scale[c] = 1/(std[c]*255) — no manual reordering. norm=None inherits the per-position defaults (layer 0: (0, 1/255); layer 1: (-127.5, 1/128)), which are the values the plate cascade of Chapter 2 runs on.

## 6.2 Four families, one contract

src/postprocess.h’s own top-of-file comment states the design: “a plain launch function, caller owns every device buffer, everything async on the caller’s stream… a different model family gets a sibling launcher next to this one, not a plugin framework.” Four families honor that contract:

| Family | Role | Decode work |
| --- | --- | --- |
| yolo | layer-0 detector | anchor decode + NMS |
| yolo-e2e | layer-0 detector | none — model emits resolved boxes |
| ctc | cascade child | greedy CTC decode (OCR) |
| argmax | cascade child | plain max-logit scan, no softmax |

argmax is also the template: a classifier postprocess simple enough (one thread per batch row, sequential scan, no probability normalization) to be the worked example for anyone adding a fifth.

## 6.3 The recipe

manual/ADDING_A_FAMILY.md turns Argmax’s seven touch points — kernel + launcher, the Family enum, graph acceptance, the GpuLoop branch, the result field, the Python _FAMILIES entry, and the bit-exact checkpoint — into a walkthrough anyone can follow without reading the whole codebase. Its §5 addendum covers what a layer-0 detector family needs beyond a layer-1 cascade child, and it exists because the recipe was followed cold to build yolo-e2e: an ExecPlan::l0_family field, both layer-0 acceptance forms widened to the new family, an explicit SAHI ruling (no NMS means no defined cross-tile merge, so SAHI is rejected by name), a different branch point in GpuLoop (Stage 1, not the cascade block), and a checkpoint that canonicalizes its own ordering before a bit-exact compare means anything. A recipe that has been used is a better recipe than one that has only been reread.

## 6.4 yolo-e2e: detection without NMS

yolo26n, a COCO-pretrained general detector, exports to an output layout of [batch, 300, 6] — [x1, y1, x2, y2, score, class_id], already sorted by score, already class-resolved, with zero NonMaxSuppression nodes in the ONNX graph (checked three independent ways: onnx.load, onnxruntime against Ultralytics’ own decode, and trtexec’s binding dump). That is end-to-end, NMS-free detection architecturally, not a yolov8 head that merely exports oddly, and the house rule — a different family gets a sibling, not a hack — makes it a family of its own. The yolo family is untouched; yolo-e2e is genuinely simpler than yolo: no anchor decode, no NMS kernel, just a score threshold over 300 pre-sorted rows and an un-letterbox.

## 6.5 The depth-2 tree

One detector root feeds N sibling recognition children — any mix of ctc/argmax — each cropping the same root detections, never each other’s output:

+--> read      (ctc)     -> texts
  detect (yolo) ----+
                    +--> classify  (argmax)  -> (label, score)

examples/read_and_classify/read_and_classify.py is exactly this: one plate detector, LPRNet OCR and a mobilenet classifier as unmodified siblings — proof the generalization is additive, since neither single-child cascade had to change. The back-compat rule matters here: r.texts/r.labels (the pre-tree fields) must equal exactly the first matching-family child’s output, not some merge of all of them. The tree-parity gate checks something stronger than alignment: run the tree once, then run read alone and classify alone as their own two-layer pipelines against the same farm content, and the tree’s dominant plate string must equal the read-only run’s, and the tree’s top argmax label must equal the classify-only run’s. Two independent single-purpose pipelines and one three-model tree, agreeing, is the actual claim being tested.

## 6.6 The classifier-parity gate

The classifier cascade demo runs cleanly — alignment is never in question — but its independent-recompute gate is not a flat agreement bar, and the reason is the content. This demo’s frames sit almost entirely on a near-tied decision between two of 1000 ImageNet classes (“digital clock” vs. “street sign”): every disagreement between the pipeline and an independent torch/cv2 reference has a small logit margin (0.003–0.098), while every confident case (margin above about 0.15) matches with no exceptions. A fixed agreement threshold or a fixed margin threshold turns into whack-a-mole against such content — a boundary-parked detection flips at margins past 0.5 depending on the run. The gate that holds is a logit-gap invariant: a disagreement is acceptable only if the reference itself scored that class within 1.0 logit of its own top pick — a boundary flip between two legitimate resizers, not a mechanics bug — plus a 60% agreement floor. With true per-channel ImageNet normalization (§6.1) raw agreement measures 78–98% on this content, and the underlying 919-vs-530 tie remains, smaller but present, which is exactly why the gate measures the margin and not a percentage. Two harness rules come with it: held frames are always released (a producer that waits on one hangs), and a crop rect is the sum of two independently rounded values, never the rounding of a sum.

Takeaway. Generality comes from a contract that keeps the same seven-touch-point shape for a fourth family (argmax), a fifth (yolo-e2e), and a tree of siblings on top of both — and from a parity gate that measures the right thing (a near-tied margin, not an arbitrary bar) where the data shows why two implementations disagree.

# Chapter 7 — Making an Atlas

## 7.1 Capacity as a function, latency as its outcome

Chapter 5’s sinks answer “can data get out without a copy tax.” A different, older question sits underneath everything the endpoint feeds: how many cameras can this actually hold, at what settings, before something gives? The project’s whole capacity model compresses into one line: capacity = f(stream count × decode rate × inference rate); latency is what falls out of that, not a fourth dial. recommend()’s two load equations make this literal — decode load is streams × decode_fps_per_stream, inference load is that same number divided again by skip — because decode and inference are independent axes: NVDEC can decode a frame nobody infers on, and skip only ever throttles the second number.

## 7.2 How a campaign is run

An atlas campaign is a grid, not a single benchmark: N ∈ {1, 4, 8, 16, 32, 50, 75, 100} streams × a decode axis (all, or keyframe-only at GOP 15/30/60/150) × an inference-skip axis (1 through 150) — roughly 42 valid (decode, inference) combinations × 8 stream counts, about 336 cells, 5–6 hours unattended per resolution. Every cell logs its own processing-time split, hold-rate, NVDEC/SM utilization, VRAM, and an auto-classified bottleneck, so a run produces a table a script can reason over, not just a folder of raw numbers. Content is real 4K plate footage downscaled per resolution, not synthetic.

Two rules of method apply to every campaign. Every verdict is stated as a prediction before the run, and every campaign opens with a control: reproduce a known cell before trusting a new one (the SAHI cells anchor on a 1.83 ms/batch no-SAHI baseline and accept ±15%), and use a cell where two code paths must agree — pooled and serial SAHI are identical at one stream — as a check that the harness itself is sound. That discipline is what makes a constant regime-scoped rather than universal: the fixed SAHI overhead measured under keyframe-only decode (2.66 ms) is not the one under every-frame decode (1.09 ms serial, 0.32 ms pooled), because the whole-frame batcher amortises it under real load. Chapter 8 applies the same rule to the atlas’s own every-frame cells.

## 7.3 The fitted models

The constants below are the ones _capacity.py carries and recommend() evaluates. §8.4 says where the every-frame capacity cells they were fitted on differ from the current decoder path; the intercepts, the keyframe cells and the SAHI fits do not. No-SAHI baseline cost, fit per resolution (gpu_ms ≈ a + b·load; only the intercept is trusted — the slope term is noise, sign-inconsistent across resolutions):

720p:  a=1.98    1080p: a=2.00    1440p: a=1.87    4k: a=2.01

All four land within ±0.07 ms — direct confirmation that inference cost is resolution-invariant, since every frame letterboxes to the same 640×640 regardless of source size.

SAHI cost, ms/frame = a + b·T (T = tile count):

regime                       a (ms)   b (ms/tile)   R^2
  keyframe-only decode          2.66     0.488        0.9978
  every frame, serial (N=8)     1.094    0.482        0.9998
  every frame, pooled (N=8)     0.323    0.498        1.0000

The slope is the same in every fit — tile compute cost does not depend on the regime. Only the fixed overhead a differs, and it differs by regime because the whole-frame batcher amortises it under every-frame decode.

Fleet ceilings (whole-GPU frames/s, 4K, at the default tile 640 and three smaller tiles):

tile   T    keyframe-only   every frame, serial (N=8)   every frame, pooled (N=8)
  1280   9    ~126            186                         208  (+65% vs keyframe-only)
  960    16   ~91             112                         121  (+33%)
  640    33   ~53             59                          60   (+13%)
  416    85   ~23             (not in this grid)          --

The shipped default (tile 640) is where pooling helps least (floor(48/33)=1 — a pooled batch essentially never carries a second frame’s tiles); most of the 640-row’s 53→60 uplift is the decode-regime difference, not pooling itself. Tile 1280 is where pooling earns its keep, at a real recall cost the atlas doesn’t measure.

## 7.4 recommend(): the atlas surfaced in the API

pycamtrt.recommend(streams, resolution, fps, skip, decode, sahi=...) turns every constant above into a live prediction, reasoning across both GPU-inference and NVDEC capacity at once and reporting whichever binds: binding_resource is "gpu-inference", "nvdec", or "none" (comfortable headroom on both), and holds_realtime is true only when both ratios clear 1.0. A worked example from its own docstring: 16 streams, 4K, tile 640 — predicted 16.757 ms/frame against a measured 16.8 in the every-frame SAHI cells is a 0.3% self-check, and the call correctly reports holds_realtime=False, suggestion='over capacity: raise skip to 9 / use decode="key" / ...' — 480 offered inference fps against a ~60 fps whole-GPU ceiling is an ~8x gap, and math.ceil(skip * gpu_ratio) says exactly 9. Every call carries the same honesty caveat, unconditionally, in its first note: every constant here was measured on one GPU (RTX 3060 Ti, sm_86); a different SM count or NVDEC generation needs its own atlas before these numbers mean anything there. recommend() is a seed, not a portable model, and it says so on every call rather than only in a docstring someone might not read. Its docstring also carries the calibration caveat §8.4 explains: above the atlas’s every-frame ceilings it is conservative until the refit on the current decoder.

## 7.5 How a user makes an atlas for their own GPU

The tooling lives in bench/atlas/.

- Farm. tools/stream_farm/farm.sh up N with real recorded content (the atlas_<res>_g30.mp4 clips bench/atlas/run_atlas.sh builds) — synthetic content is a known trap; the 89-cell partial run on plate-over-webcam is why it’s not used anymore.

- Build the resolution. bench/atlas/run_atlas.sh <res> smoke first (one sanity cell), mini for the ~30-cell validation grid, then full for the 336-cell grid — one resolution to completion before starting the next; runs resume from their own atlas.csv.

- Grid. Let the orchestrator sweep N × decode × skip; each cell writes its own row (processing splits, utilization, VRAM, bottleneck label) to atlas.csv.

- Fit. python3 bench/atlas/atlas_analyze.py <run>/atlas.csv turns cells into a cost model — the baseline intercept, the NVDEC-utilization ceiling.

- Constants. Copy the fitted intercepts/slopes into pycamtrt._capacity’s tables, cited inline to the analysis file they came from, and re-run qa_matrix.py section H — its self-check against a real measured cell is the same 0.3%-style anchor that caught nothing wrong here, and would catch a transcription error.

Takeaway. An atlas is a grid run with a prediction stated before every cell and a control built into every campaign — that discipline is what shows a constant to be regime-scoped rather than wrong. recommend() is that whole method compressed into one function call, with its single-GPU limitation stated on every result it returns.

# Chapter 8 — The Decoder Path and the Ceiling

## 8.1 How a frame reaches the batch

Everything above this chapter treats decode as a solved input. This chapter walks the path a frame takes from the network to the detector’s input tensor, because three details of that path decide where the fleet ceiling sits, and each is short enough to miss.

One producer thread per camera runs the loop. It demuxes the next packet on the CPU and, if a relay or clip sink is configured, copies it into the packet ring of Chapter 5. It feeds the packet to the NVDEC parser with the end-of-picture flag set: every packet the demuxer hands over is a whole access unit, so the parser can decode it on arrival instead of waiting for the next packet’s start code, which at a live camera would arrive one frame period later. The parser calls back into the same thread: “decode this picture”, submitted to the hardware, and “picture ready”, where the producer maps the decoded surface. The map does more than return a pointer; it runs a post-processing pass that turns the decoder’s tiled surface into the pitch-linear NV12 buffer the next kernel reads, and it runs that pass on the stream the producer names in the map’s parameter struct — the producer’s own non-blocking, highest-priority CUDA stream, the same one the letterbox kernel will use, so ordering between them is plain stream order. Every decoder owns its own NVDEC context lock, as NVIDIA’s decoder sample does, so a camera’s submit, map and unmap contend with nothing but themselves.

The producer then takes a slot in its camera’s NV12 ring and a slot in the batch buffer being filled, launches the letterbox kernel straight into that slot and a full-frame copy into the ring slot for the cascade’s crops, records an event, sleeps until both have finished, unmaps the surface, and marks the slot ready. Three batch buffers rotate: one filling, one in the GPU thread, one spare, so producers never wait for the GPU in normal operation. The GPU thread takes the oldest full buffer, or closes the filling buffer the moment anything in it is ready and waits only for slots already handed out — there is no batch timeout — then runs the detector, the GPU postprocess, the crops and the child engines, pushes one result per slot, and recycles the buffer. Under light load the batches are one or two frames, which is where the 2 ms comes from; under heavy load they grow to whatever became ready during the previous cycle.

Leave out two of the three details and the library measures very differently, which is why they are worth spelling out. With the map’s stream field left at zero, the post-processing ran on the legacy default stream, which waits for every earlier kernel on every blocking stream in the context — including the TensorRT batch in flight on the inference stream. Invisible at sixteen cameras, where a batch takes two milliseconds; at thirty-two, where a batch takes ten, every map waited for it. With one lock shared by all cameras and held during that map, every other camera then waited for the waiting one, the hardware decoder ran dry, and the GPU thread waited for the stragglers the lock released in a burst. The counters read half everywhere — SM 54%, decoder 52%, memory controller 43% — while the pipeline’s own timers showed a frame waiting 38 ms between decode and result at a 33 ms frame period. The full-inference ceiling then sits at 24 cameras at 720p and 16 at 1080p, half the hardware’s, and neither half of the cause shows up on its own: kernel starvation and the lock by itself each test clean, because each is harmless without the other. With the stream field set and one lock per decoder the ceiling is the decoder itself, and a race goes with it: a letterbox kernel on a non-blocking stream never waits for post-processing on stream zero, and can read a surface still being written, so detections drift run to run. On the producer’s stream they are byte-identical, and bench/smoke_digest.py is the gate that says so.

The end-of-picture flag is the third detail. Measured with a timer from demuxer to pop that no result field covers (PYCAMTRT_DIAG_DEMUX=1 prints it at exit), a 1080p frame at 24 cameras spends 53 ms between leaving the demuxer and being popped without the flag and 19 ms with it; at 720p and 48 cameras, 46 and 10 ms. The remainder is the decoder’s own queue, which grows only when NVDEC saturates. Letting the parser run a picture ahead per camera, the other decoder-side lever, buys no throughput at the wall (740 fps against 738 at 1080p) and costs the frame period it queues, so the parser runs with display delay zero.

## 8.2 Where the ceiling is

With the path as described, the fleet stops where the hardware does. The same-night campaign of Chapter 4 put the decoder counter on every system:

| every frame at 30 fps, holds ≥95% | pycamtrt | where it stops |
| --- | --- | --- |
| 720p | 48 cameras (end of the campaign grid; the atlas grid holds 50 at 11.8 ms) | decoder 97–100%, GPU 94% |
| 1080p | 24, 3.3 ms; 28 reaches 88% | decoder 99–100% |
| 1440p | 8, 2.0 ms; 16 reaches 86% | decoder 100% |
| 4K | 6, 1.8 ms; 8 reaches 80% | decoder 100% |

Tuned DeepStream stops at the same counts with the same counter readings: at the 1080p wall it decodes 715 fps to pycamtrt’s 737, both at 100%, and at 4K 193 to 192. Two walls arrive together at 720p, the decoder at 97% and the GPU at 94%, which is why 48 to 50 cameras is the end there for either system on this card; everywhere else the decoder alone is the limit. What pycamtrt keeps below the wall is the latency of Chapter 4 — equal at one camera, two to four times lower from a few cameras up to the wall, equal on it. Pop-to-ready, the producer’s own share, reads 0.05 ms at every count.

## 8.3 The dials against DeepStream

DeepStream exposes the same two capacity dials pycamtrt does: interval on its primary engine is the skip, and intra-decode-enable on a source is keyframes-only. Run on the same grid, farm, content and hour as the validation atlas of §8.4:

| cameras held, pycamtrt / DeepStream tuned | 30, 15, 10, 5, 2 inferences/s per camera, every frame decoded | keyframes only, 1/s |
| --- | --- | --- |
| 720p | 50 / 50 | 100 / 100 |
| 1080p | 16 / 16 | 100 / 100 |
| 1440p | 8 / 8 | 75 / 75 |
| 4K | 4 / 4 | 32 / 32 |

Identical at every resolution and rate, and for a physical reason: every-frame decode ends where NVDEC saturates for both, and keyframe-only decode at 1440p and 4K ends where VRAM runs out for both. The inference dial changes GPU load — at 720p and 50 cameras from 94% at full rate to 18% at two inferences per second — but never capacity, on either side, because the decoder is the wall.

What differs below the wall is how long a frame waits, and DeepStream’s own component split says where. Its muxer holds frames until a batch of N is complete or 5 ms pass; with eight cameras arriving at eight phases that is 4.9 ms of waiting per frame before inference begins, shrinking to 1.9 ms at 32 cameras as batches fill faster. Its primary and secondary engines are separate elements with their own queues and threads, 3 to 6 ms together against pycamtrt’s 2 to 3 for the same cascade. pycamtrt closes a batch the moment the GPU thread is free and anything is ready, and runs the cascade inside one thread on one stream: its queue term is 0.2 to 1.5 ms. Same silicon, same walls, different waiting. DeepStream’s latency under its skip dial averages inferred and skipped frames alike, so latency is compared at full rate only; the figure in the README states both measurement definitions in its footnote.

## 8.4 The atlas fits and the current decoder

The fits behind recommend() (§7.3) come from an atlas measured with the map on the default stream and one lock shared by every decoder — the path §8.1 describes as the one without two of its details. That leaves the intercepts, the keyframe cells and the SAHI fits valid, and the every-frame capacity cells above the old ceilings conservative. A 96-cell validation grid on the current decoder — the every-frame cells around the ceilings plus keyframe and single-camera anchors at all four resolutions — puts the two side by side; eleven cells hold on the current decoder that the fits call over capacity, and none goes the other way:

| every frame, skip 1 | atlas fits | current decoder |
| --- | --- | --- |
| 720p | 32 cameras at 61 ms | 50 at 11.8 ms |
| 1080p | 16 | 16 (32 reaches 76%) |
| 1440p | 8 | 8 (16 reaches 86%) |
| 4K | 4 | 4 (8 reaches 78%) |

Every keyframe cell reproduces — 100 cameras at 1080p at 99% with the decoder at 16% — and so does the VRAM wall at 1440p and 4K with 100 keyframe cameras. recommend()’s docstring states the consequence: above the atlas’s every-frame ceilings it is conservative, reporting over-capacity where the current build holds, until the full sweep on this decoder refits it. The atlas tooling in bench/atlas/ runs that sweep; bench/atlas/atlas_vs_deepstream.py reproduces the comparison of §8.3.

## 8.5 What the ceiling teaches

A utilization counter at half is a symptom of waiting, not a bottleneck. Three counters at 43–54% with a plateau underneath them meant the machine was idle in turns, and the only instrument that could say so was the pipeline’s per-frame timers.

A suspect cleared twice can be guilty in combination. The shared lock was harmless alone and the map on stream zero was harmless at sixteen cameras; together, past the point where a batch outlasts a frame period, they were the ceiling.

Read the vendor’s sample against the wrapper. The field that mattered is set in NVIDIA’s sample, vendored in the tree.

Determinism is a free gate. Hashing detections on a file found a race no accuracy check had, because the race never changed a count, only a decimal.

Measure everything in one sitting. A DeepStream decode figure from a different night suggested an 8% gap that did not exist when both systems ran an hour apart.

Takeaway. The full-inference ceiling is the hardware decoder, for pycamtrt and for DeepStream alike: 48/24/6 cameras with every frame at 720p/1080p/4K, 100/100/32 with keyframes only. What pycamtrt keeps is the wait below that wall — two to four times shorter — and detections that are byte-identical run to run.

Cost so far. Three decoder-side details — the map on the producer’s stream, a lock per decoder, the end-of-picture flag — and 0.05 ms of producer time per frame; capacity identical to DeepStream on every dial, latency two to four times lower below the wall.

# Chapter 9 — Where It Stands, What’s Next

## 9.1 What exists, and its gate

The house rule holds without exception: nothing here is “done” without a gate that can be re-run. The table below is the whole surface, each row with the specific check that would fail first if the row stopped being true.

| Feature | Gate |
| --- | --- |
| Endpoint sinks (events / stream relay / clip) | qa_matrix F1–F3; 3.5 h soak, 0 drops |
| SAHI tiled inference (pooled default, serial A/B) | tools/sahi_parity.sh, self-controlled noise floor |
| Families: yolo, yolo-e2e, ctc, argmax | bit-exact postprocess_batch_test.cpp checkpoints; qa_matrix G/I1 |
| Depth-2 tree (N sibling children) | qa_matrix I2 tree-parity (dominant-plate + top-label cross-checks) |
| Per-channel norm/color | crop_test, 24/24 bit-exact |
| Per-stream skip/decode overrides | qa_matrix C/D |
| Drop-oldest backpressure | held 96% real-time pace under a stalled consumer, 788 evicted, verified |
| Three-tier frame access (addr/fetch/cuda) | pipeline_api_test lifecycle (0 MB VRAM growth, 20-cycle matrix) |
| recommend() capacity planner | qa_matrix H (4 sub-checks incl. a measured-cell self-check) |
| fp16 dual-stage cascade | agreement-verified: dominant plate identical, minority misread 2.3%→4.8% |
| The decoder path of Chapter 8 | bench/smoke_digest.py byte-identical across runs; pipeline_api_test 5-cycle lifecycle, VRAM flat; reconnect churn; HEVC; same-night ceiling campaign |
| End-of-picture packet flag | demuxer-to-pop 19 ms at 1080p/24 (53 without it), digests unchanged |
| pip install . from source | builds in the container without git; import and uninstall verified |

## 9.2 The honest gaps

A feature table only tells half the story if the missing half stays implicit.

| Gap | State |
| --- | --- |
| No object tracker | skipped frames carry no boxes; skip>1 has no interpolation — an open design decision |
| No multi-GPU | one primary CUDA context; every recommend() constant is single-GPU (RTX 3060 Ti) only |
| No runtime dynamic stream add/remove | fleet size is fixed at Pipeline construction |
| hold_frames (tier 3) | an exit-time crash seen in earlier internal builds has not reproduced across 140 hunt attempts or the lifecycle checkpoint, but its write site was never localized — treated as suppressed, not proven fixed |
| recommend() calibration | fitted on the atlas of Chapter 7: conservative above ~24 cameras at 720p / 16 at 1080p with every frame decoded, until the refit on the current decoder (§8.4) |
| No INT8 path in the builder | fp16 only; a prebuilt INT8 engine can be loaded, but calibration is the user’s job — the next lever at 720p, where the GPU sits at 93% with 48 cameras |
| LPRNet charset | Chinese — mechanics-correct, wrong alphabet for EU plates; a content swap, not an architecture change |

## 9.3 The roadmap, in order

- Tracker + skip as one feature. The largest capability gap: skip=2 already frees half the GPU, but produces no continuous identity across skipped frames. The owner decision still open is skipped-frame semantics — track IDs on inferred frames only, or also emit flagged predicted boxes on skipped ones.

- NVENC annotated sink. The one case Chapter 5’s answer table couldn’t dodge: draw in VRAM, encode, republish — the Sink graph node already built is the base it attaches to.

- Per-track OCR voting. The systematic fix for fp16’s read jitter (the 2.3%→4.8% minority misread from §9.1) — needs the tracker first, since “per-track” presumes identity across frames.

- Plugin ABI. The compiled-family path (Chapter 6) is a deliberate scope limit — no dlopen, no stable C ABI — deferred until a family finds the seven-touch-point recompile cost actually prohibitive in practice, not before.

- Packaging. pip install . via scikit-build-core works inside the documented container; a prebuilt wheel, closing the build-inside-docker requirement, does not exist (TensorRT redistribution limits apply regardless of packaging).

- The atlas refit. The full 336-cell grid per resolution on the current decoder, then _capacity.py refitted from it, so recommend() stops under-promising above the atlas’s every-frame ceilings. Then INT8 for the detector, both here and on the DeepStream side, for an equal comparison at 720p where compute is now the second wall.

- Deeper trees. The executor stops at depth-2 by name (Validate()’s own error names the limit); a chain executor is future work the API already expresses without yet running.

## 9.4 The repository and its versions

pycamtrt lives in its own repository on its own release line — v0.4.0 is the first public release — separate from the research engine’s history, which is not redistributed. The C++ core keeps its identity underneath (pycamtrt_core), and the two audiences the design serves — an engine contributor building kernels, a Python user declaring a graph — read the same repository through src/ and python/ respectively. CHANGELOG.md records what each version added; the version string is kept in lockstep in pyproject.toml, CMakeLists.txt and the package.

## 9.5 Afterword: one method

Part 1 proved a claim: a hand-built pipeline could hold its own frame budget, verified bit-exact against a CPU reference at every stage. Part 2 widened the unit of study from one frame to a fleet, and from a claim to a comparison. This part turns the same three through-lines — zero-copy, cost accounting, verification — outward: an endpoint that sends data back out without a copy tax, a family system proven general by being used to add new model shapes, a capacity atlas turned into a function anyone can call before they buy a GPU, and a decoder path whose ceiling is the silicon and not a wait.

Chapter 8 is the method turned on the library’s own results, with the instruments the series is built from: per-frame timers instead of utilization counters, a same-night grid instead of numbers from two dates, a digest instead of a glance. The habit of re-measuring what is already “known” is the expensive part, and the part worth keeping.

Takeaway. Every feature in this book ships with a gate, and every gap is stated as plainly as every feature — a library earns trust the same way a single kernel does: not by claiming completeness, but by being honest about exactly where it ends.